- Appendix E

- Maps and Additional Study

- Information

Appendix E

Maps and Additional Study Information

Table of Contents

Page

1.	Prior Studies and Reports	E-2

2.	Description of Existing Federal Project	E-4

3.	Existing Channel Structures (Dikes/Revetments)	E-6

4.	New and Modified Channel Structures	E-13

5.	Notched Dikes/Revetments	E-22

6.	Maintenance Dredging History	E-24

7.	Future Anticipated Dredging Needs	E-25

8. 	Existing Levees in the Study Area	E-26

List of Tables

Table E-1	Navigation Pools of the MKARNS	E-4

Table E-2	Eleven Principal Upstream Storage Reservoirs in the

Arkansas River Basin	E-5

Table E-3	Existing Channel Structures in Oklahoma	E-6

Table E-4	Existing Channel Structures on the MKARNS in Arkansas by Pool	E-7

Table E-5	Structures to be Raised, Extended, or Added in Arkansas	E-14

Table E-6	Notched Dikes/Revetments	E-22

Table E-7	Maintenance Dredging History on the Verdigris River	E-24

Table E-8	Maintenance Dredging History on the Arkansas River	E-24

Table E-9	Future Anticipated Dredging on the Verdigris River	E-25

Table E-10	Future Anticipated Dredging on the Arkansas River	E-25

Table E-11	Levees Within the ARNS study area	E-26

# Maps

Arkansas River Navigation Study Maps, Sheets 1-95

1.  Prior Studies and Reports

Listed below are some prior reports about the study area in Arkansas:

(1)  McClellan-Kerr Arkansas River Navigation System (MKARNS), Potential for Increased Channel Depths, Initial Appraisal Report, U.S. Army Corps of Engineers, Little Rock District, Operation and Maintenance Funds, October 1998.  The report recommendation was for a reconnaissance study to be conducted to determine the economic feasibility of increasing the channel depth of the MKARNS.  The report was approved in November 1998 for a proposed new reconnaissance start beginning with FY 2001.

(2)  Rice-Carden Levee, Fort Smith, Arkansas, Section 205, Detailed Project Report and Environmental Assessment, U.S. Army Corps of Engineers, Little Rock District, June 1993.  The report was never approved because it was found to be in violation of Executive Order (E.O.) 11988;.the study was terminated in 1994.

(3)  May Branch, Fort Smith, Arkansas, Section 205, Reconnaissance Report, U.S. Army Corps of Engineers, Little Rock District, March 1992. The feasibility study cost sharing agreement (FCSA) was signed in November 1998, and the feasibility study is scheduled for completion in September 2001.  Total feasibility study cost is $1,480,000; the city's share is $740,000.

(4)  Arkansas River Basin, Arkansas and Oklahoma, Feasibility Report, U.S. Army Corps of Engineers, Little Rock District, May 1991.  Several operating plans on the MCKARNS were investigated, and the report recommended further investigations.  However, the issues associated with those operating plans were never resolved due to the impacts of those plans on floodplain lands and improvements along the Arkansas River.

(5)  Arkansas River Basin, Arkansas and Oklahoma, Feasibility Study, Operational Plans Review Status Report, U.S. Army Corps of Engineers, Little Rock District, March 1988.  This report was part of the Arkansas River Basin Feasibility Study and presented a status of the navigation studies and identified and compared the impacts of three operating plans on the MKARNS.

(6)  Arkansas River Basin, Arkansas and Oklahoma, Reconnaissance Report, U.S. Army Corps of Engineers, Little Rock District, August 1986.  The purpose of this report was to evaluate the need and opportunities for reducing flood damages and for developing additional municipal, industrial, and water supplies in the Arkansas River Basin.  Later on the objectives were expanded to evaluate the potential for improvements to the MKARNS.  The report recommended that feasibility studies be conducted.

(7)  Mill Creek, Fort Smith, Arkansas, Section 205, Detailed Project Report, U.S. Army Corps of Engineers, Little Rock District, June 1985.  The report recommended a channel improvement project that was constructed in 1992 at a total cost of almost $9,000,000.

(8)  Little Massard Creek, Fort Smith, Arkansas, Section 205, Detailed Project Report, U.S Army Corps of Engineers, Little Rock District, June 1983.  The report recommended a channel widening and bridge alteration project that was constructed in 1988 at a total cost of $191,000.

(9)  Reconnaissance Report Fort Smith-Van Buren Study, General Investigation, U.S. Army Corps of Engineers, Little Rock District, November 1980.  The report recommended that the flooding problems be separated into individual Section 205 studies.

Listed below are some prior reports about the study area in Oklahoma:

(1)  Bank and Channel Stabilization of the Arkansas River from Keystone Lake to Webbers Falls Lock & Dam, Oklahoma / Feasibility Report, U.S. Army Corps of Engineers (Tulsa District), December 1977.

(2)  The waterway: a report on the Arkansas River navigation system and regional growth, Kerr Foundation: Economic Studies Division - (Oklahoma City), 1977.

(3)  Feasibility Report for Extending Navigation from the Port of Catoosa to Vicinity of Wichita, KS, U.S. Army Corps of Engineers (Tulsa District), January 1975.

(4)  Coal Electricity and Navigation for the Mid Arkansas Region (1990-2040), Richard J. Bigda & Associates, February 1974

(5)  Lock and Dam No. 14, Oklahoma: Design Memorandum No. 10, Navigation Channel, Pools 13 and 14, U.S. Army Corps of Engineers (Tulsa District), July 1967.

(6)  Towboats A’ Commin’ [Motion Picture], U.S. Army Corps of Engineers - Released by National Audiovisual Center, 1967.

(7)  Extension of Arkansas River Navigation to S. Central KS, Mid Arkansas Valley Development Association, Inc. (Wichita, KS), 1966.

(8)  Survey Report on Central Oklahoma Project - Special Economic Studies, U.S. Army Corps of Engineers (Tulsa District), March 1965.

(9)  Central Oklahoma Project, U.S. Army Corps of Engineers (Tulsa District), March 1965.

(10)  Lock and Dam No.16, Webbers Falls, Arkansas River, Oklahoma: Design Memorandum No.10, Navigation Lock, U.S. Army Corps of Engineers (Tulsa and Buffalo Districts), November 1964.

(11)  Lock and Dam No.15, Robert S. Kerr, Arkansas River, Oklahoma: Design Memorandum No.14, Navigation Lock, U.S. Army Corps of Engineers (Tulsa and Buffalo Districts), May 1964.

(12)  Navigation and Sedimentation Conditions at Typical Lock and Dam, Arkansas River, Arkansas and Oklahoma: Hydraulic Model Investigation, U.S. Army Engineer Waterways Experiment Station, Corps of Engineers, Vicksburg, MS., 1963.

(13)  The Effect of Proposed Extensions of the Arkansas River Navigation Project on Utilization of Southeastern OK & West AR Coals, Arthur D. Little Inc., October 1962.

2.  Description of Existing Federal Project

- Within each reservoir, three zones of water storage are present consisting of the flood control pool, the conservation pool, and the inactive pool.  The flood control pool is the upper pool and is reserved for retaining floodwaters.  It is used only during flood control periods.  The conservation pool in the middle zone provides water for power generation, flow regulation, and water supply.  The bottom pool or inactive pool provides water pressure for water release and power generation as well as sediment trapping.  Water storage is measured in acre-feet, which is the amount of water available to cover 1 acre to a depth of 1 foot.

- Table E-1 summarizes the characteristics of each navigation pool.

| Table E-1 Navigation Pools of the MKARNS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Navigation Pool (NP) | Length (miles) | Surface Area | Capacity | Navigation Mile | Elevation |
| Oklahoma Pools |  |  |  |  |  |
| Newt Graham NP | 23.2 | 1,490 | 2,500 | 421.6 | 532 to 511 |
| Chouteau NP | 20.2 | 2,270 | 23,340 | 401.4 | 511 to 490 |
| Webbers Falls Lake | 32.5 | 11,640 | 170,100 | 368.9 | 490 to 460 |
| Robert S. Kerr Lake | 32.7 | 32,800 | 525,700 | 336.2 | 460 to 412 |
| W.D. Mayo NP | 16.6 | 1,595 | 15,800 | 319.6 | 412 to 392 |
| Arkansas Pools |  |  |  |  |  |
| Hammerschmidt Lake (J.W. Trimble) | 26.8 | 6,820 | 59,100 | 292.8 | 392 to 372 |
| Ozark Lake (Ozark-Jeta Taylor) | 36.0 | 10,600 | 148,400 | 256.8 | 372 to 338 |
| Dardanelle Lake | 51.3 | 34,300 | 486,200 | 205.5 | 338 to 284 |
| Rockefeller Lake (Arthur V. Ormond) | 28.6 | 5,660 | 64,600 | 176.9 | 284 to 265 |
| Toad Suck Ferry NP | 21.0 | 4,130 | 33,000 | 155.9 | 265 to 249 |
| Murray NP | 30.5 | 9,700 | 87,100 | 125.4 | 249 to 231 |
| David D. Terry NP | 17.3 | 4,710 | 49,500 | 108.1 | 231 to 213 |
| Lock & Dam No. 5 NP | 21.8 | 6,680 | 61,300 | 86.3 | 213 to 196 |
| Emmett Sanders NP | 20.3 | 5,680 | 70,400 | 66.0 | 196 to 182 |
| Joe Hardin NP | 15.8 | 3,670 | 46,400 | 50.2 | 182 to 162 |
| No. 2 | 36.9 | 10,600 | 110,080 | 13.3 | 162 to 142 |
| Norrell | 3.1 | 140 | 1,510 | 10.2 | 142 to WR |

Table E-2 lists the 11 upstream reservoirs and their corresponding flood control storage.

| - Table E-2 - Eleven Principal Upstream Storage Reservoirs in the Arkansas River Basin |  |  |
| --- | --- | --- |
| - Project | - River | - Flood Storage - (Acre-Feet) |
| - Keystone | - Arkansas | - 1,180,000 |
| - Oologah | - Verdigris | - 965,600 |
| - Pensacola | - Grand (Neosho) | - 525,000 |
| - Hudson | - Grand (Neosho) | - 244,200 |
| - Fort Gibson | - Grand (Neosho) | - 919,200 |
| - Tenkiller | - Illinois | - 576,700 |
| - Eufaula | - Canadian | - 1,510,800 |
| - Kaw | - Arkansas | - 919,400 |
| - Hulah | - Caney | - 257,900 |
| - Copan | - Little Caney | - 184,300 |
| - Wister | - Poteau | - 386,800 |
| - Total |  | - 7,669,900 |

3.  Existing Channel Structures

Table E-3 summarizes the existing channel structures in Oklahoma.

| Table E-3 Existing Structures in Oklahoma |  |  |
| --- | --- | --- |
| Location | Estimated Number | Type of Structure |
| Left RM 308.6-RM309 | 8 | Dikes/Revetments |
| Left RM 309.6-RM10.3 | 3 | Dikes/Revetments |
| Left RM 315.4-RM-317.3 | 9 | Dikes/Revetments |
| Left RM 317.3-318.0 | 9 | Dikes/Revetments |
| Right RM 319.2-319.5 | 3 | Dikes/Revetments |
| Left RM320-321.1 | 6 | Dikes/Revetments- backwater areas |
| Left RM 321.2-RM323.3 | 7 | Dikes/Revetments- backwater areas |
| Right RM 321.8.2-323.5 | 9 | Backwater area |
| Left RM 326.3 -328.0 | 1 | Backwater area |
| Left RM 328.5-328.8 | 1 | Dikes/Revetments |
| Right RM 330.4-330.7 | 1 | Dikes/Revetments |
| Right RM 332.4-332.7 | 1 | Four Dikes/Revetments |
| Right RM 355.4-RM356.4 | 1 | Dikes/Revetments Confluence with Canadian |
| Canadian River | 1 | Long Dikes/Revetments |
| Left RM363.3-RM364 | 3 | Dikes/Revetments |
| Left RM367.7-370.9 | 1 | Rip Rap |
| RM393.2-RM395 | 1 | Dikes/Revetments Field (Muskogee-3 Forks Area) |
| Right RM394.0-RM394.4 | 3 | Dikes/Revetments |
| Right up Arkansas Confluence | 8 | Dikes/Revetments |
| Left up Arkansas Confluence | 8 | Dikes/Revetments |
| Left RM433.7-RM434.2 | 1 | Dikes/Revetments |

Table E-4 summarizes the existing channel structures in Arkansas.

| Table E-4 Existing Structures on the MKARNS in Arkansas by Pool |  |  |
| --- | --- | --- |
| Pool | Navigation Mile* | Number of Structures |
| Arkansas In-Stream Structures |  |  |
| 2 | 19.2-20.18R | 4 |
| 2 | 22.6R | 1 |
| 2 | 23.0-28.28L | 13 |
| 2 | 27.78-33.32R | 20 |
| 2 | 31.65-36.60L | 21 |
| 2 | 35.05R | 2 |
| 2 | 37.20-40.01R | 17 |
| 2 | 37.55-40.1L | 22 |
| 2 | 41.85-44.17L | 9 |
| 2 | 42.84-43.78R | 8 |
| 2 | 45.37-45.72L | 5 |
| 2 | 45.55-46.73R | 8 |
| 2 | 46.66-50.05L | 14 |
| 2 | 47.9R | 1 |
| 2 | 48.5-50.2R | 14 |
| 3 | 50.42-50.88L | 6 |
| 3 | 50.65-54.37R | 14 |
| 3 | 53.58L | 1 |
| 3 | 54.52-56.23R | 16 |
| 3 | 55.12L | 1 |
| 3 | 56.62-56.67L | 2 |
| 3 | 57.2R | 1 |
| 3 | 58.25L | 1 |
| 3 | 58.52-59.35R | 4 |
| 3 | 60.12-62.15L | 16 |
| 3 | 60.12-61.14R | 9 |
| 3 | 62.62-63.6R | 6 |
| 3 | 63.61-65.38L | 8 |
| 3 | 64.36-65.7R | 7 |
| 4 | 68.88-71.1R | 7 |
| 4 | 69.5-70.7L | 14 |
| 4 | 73.75-74.42R | 6 |
| 4 | 75.26-75.55L | 4 |
| 4 | 76.8R | 1 |
| 4 | 78.82-82.57L | 13 |
| 4 | 78.47-81.4R | 9 |
| 4 | 82.88-86.19R | 21 |
| 4 | 86.0-86.08C | 1 |
| 5 | 88.55-89.15L | 5 |
| 5 | 91.6-92.34R | 5 |
| 5 | 91.7-92.4L | 4 |
| 5 | 93.2-93.6L | 5 |
| 5 | 94.3-95.9L | 5 |
| 5 | 98.7-99.5R | 5 |
| 5 | 99.8-108.0L | 44 |
| 5 | 102.4-104.7R | 8 |
| 5 | 107.3-107.8R | 3 |
| 6 | 108.5-110.0R | 11 |
| 6 | 111.5L | 1 |
| 6 | 112.27-112.9R | 9 |
| 6 | 112.9-115.82L | 13 |
| 6 | 114.55-118.7R | 21 |
| 6 | 117.55-121.6L | 13 |
| 6 | 120.16-120.92R | 6 |
| 6 | 122.5-124.8L | 8 |
| 6 | 122.95-125.05R | 12 |
| 7 | 125.5-126.25R | 10 |
| 7 | 126.5-128.14L | 10 |
| 7 | 128.05-133.0R | 20 |
| 7 | 129.6-130.85L | 4 |
| 7 | 133.0-136.4L | 13 |
| 7 | 133.92-138.0R | 8 |
| 7 | 136.98-139.12L | 8 |
| 7 | 139.7-143.2R | 14 |
| 7 | 145.7-151.3L | 22 |
| 7 | 146.13-149.28R | 11 |
| 7 | 151.2-155.7R | 21 |
| 7 | 153.0-155.6L | 7 |
| 8 | 156.05-156.3L | 6 |
| 8 | 156.7-156.92R | 3 |
| 8 | 158.13-160.15L | 11 |
| 8 | 158.9-160.85R | 5 |
| 8 | 161.2-167.0L | 18 |
| 8 | 162.82-163.48R | 5 |
| 8 | 165.4-169.4R | 11 |
| 8 | 169.27-176.33L | 23 |
| 8 | 171.08-176.6R | 25 |
| 9 | 178.2-185.3R | 21 |
| 9 | 179.75-180.25L | 3 |
| 9 | 186.1-186.5L | 3 |
| 9 | 186.95-193.42R | 24 |
| 9 | 188.9-190.28L | 9 |
| 9 | 193.48-196.92L | 11 |
| 9 | 196.45-198.15R | 7 |
| 9 | 198.7-203.37L | 17 |
| 9 | 201.54-205.1R | 16 |
| Lake Dardanelle (Pool 10) | 234.0-236.3R | 5 |
| 10 | 236.42-241.05L | 13 |
| 10 | 241.5-244.53R | 10 |
| 10 | 244.25-246.91L | 12 |
| 10 | 247.1-249.64R | 14 |
| 10 | 250.75-256.15R | 16 |
| 10 | 251.7-256.2L | 16 |
| Ozark Lake (Pool 12) | 274.78-276.5R | 8 |
| 12 | 275.15-276.75L | 5 |
| 12 | 279.3-280.48L | 5 |
| 12 | 279.58-280.35R | 5 |
| 12 | 282.18-283.46L | 4 |
| 12 | 282.25-283.57R | 9 |
| 12 | 285.4-287.13R | 7 |
| 12 | 285.65-289.88L | 24 |
| 12 | 289.35-291.56R | 11 |
| 12 | 292.05-292.35L | 3 |
| 13 | 293.15-294.23L | 5 |
| 13 | 294.35-295.05R | 5 |
| 13 | 296.4-297.42L | 6 |
| 13 | 297.74-300.35R | 8 |
| 13 | 300.65-301.16L | 3 |
| 13 | 301.4-305.08R | 19 |
| 13 | 302.45-308.65L | 18 |
| 13 | 305.98-306.65R | 5 |
| Oklahoma In-stream Structures |  |  |
| 13 | 308.7L | 1 |
| 13 | 309.4-311.4L | 11 |
| 13 | 309-310R | 4 |
| 13 | 311.4L | 1 |
| 13 | 313.7-314L | 2 |
| 13 | 315-319L | 27 |
| 13 | 313.6-315.2R | 7 |
| 13 | 316.2-317.8R | 8 |
| 13 | 318.4-319.5R | 8 |
| 13 | 319.3L | 1 |
| W.D. Mayo Lake (Pool 14) | 319.8-322.2 L | 11 |
| 14 | 320.1-320.8R | 6 |
| 14 | 321.3-323.9R | 14 |
| 14 | 322.7-324.3L | 9 |
| 14 | 324.8-325.4L | 2 |
| 14 | 326.3-329L | 6 |
| 14 | 330-335.2L | 19 |
| 14 | 326-328.7R | 13 |
| 14 | 330.7-331.2R | 9 |
| 14 | 332.4-332.7R | 4 |
| Robert S. Kerr Lake (Pool 15) | 340.6-341.1L | 3 |
| 15 | 345L | 1 |
| 15 | 358.9-359.6L | 1 |
| 15 | 360.5-360.6L | 2 |
| 15 | 363.2-364L | 3 |
| 15 | 355.8-358.3R | 4 |
| 15 | 360.6-361.1R | 6 |
| Webber Falls (Pool 16) | 392.2-392.9R | 12 |
| 16 | 394.3-394.9R | 4 |
| 16 | 400.2-400.3R | 1 |
| 16 | 366.7-367.7L | 2 |
| 16 | 393.4-393.9L | 4 |
| Newt Graham Lake (Pool 18) | 433.7-434.1L | 1 |
| * Navigation miles upstream from the mouth of the White River (WR). Source: MKARNS Navigation Charts, 1997 and USACE, 2004. |  |  |

4.  New and Modified Channel Structures

Table E-5 lists the structures to be raised, extended, or added to support a 12-foot channel in Arkansas.   Refer to Table C-4 in Appendix C, Engineering – Design & Cost, for a list of the new structures needed to support a 12-foot channel in Oklahoma.

Table E-5

### Structures to be Raised, Extended, or Added in Arkansas

To Support a 12-Foot Navigation Channel

| Navigation Mile | Structure Number | Existing Type (*1) | Construction Activity | Typical Section (*2) (Type #) Description | Existing Ground Elevation FT-NGVD | Existing Top Elevation FT-NGVD |  |  | Proposed Top Elevation FT-NGVD | Delta Length FT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POOL 2 – REACH 1 |  |  |  |  |  |  |  |  |  |  |
| 24.2-24.7 | 47.3L | R | Raise | (1) Dike | 146.0 | 163.0 |  | 164.0 |  | 0 |
| 28.1-28.5 | 50.2L | R | Extend | (2) Pile Dike | 148.0 | 156.4 |  | 166.0 |  | 750 |
| 27.9 | 49.8R | D | Raise | (1) Dike | 147.0 | 162.0 |  | 166.0 |  | 0 |
| 28.2 | 50.1R | D | Raise | (1) Dike | 155.0 | 162.0 |  | 166.0 |  | 0 |
| 28.6 | 50.5R | D | Raise | (1) Dike | 154.0 | 159.0 |  | 166.0 |  | 0 |
| 31.8-33.2 | 53.7-54.7L | R | Raise | (1) Dike | 158.0 | 163.0 |  | 167.0 |  | 0 |
| 33.2 | 54.7L | D | Raise | (1) Dike | 157.0 | 162.5 |  | 167.0 |  | 0 |
| 36.15 | 57.4L | D | Extend | (1) Dike | 155/145 | 164.0 |  | 168.5 |  | 100 |
| 36.3 | 57.6L | D | Extend | (2) Pile Dike | 155/145 | 160.0 |  | 168.5 |  | 100 |
| 36.5 | 57.7L | D | Extend | (2) Pile Dike | 155/145 | 160.0 |  | 168.5 |  | 100 |
| 37.4-38.45 | 58.85-59.5L | D | Raise | (1) Dike | 148.0 | 153.0 |  | 169.0 |  | 0 |
| 39.15-39.55 | 60.7L | R | Raise | (1) Dike | 155.0 | 163.5 |  | 169.5 |  | 0 |
| 39.55-39.8 | 60.9L | R | Raise | (1) Dike | 155.0 | 165.0 |  | 169.5 |  | 0 |
| 39.8 | 60.9L | D | Raise | (1) Dike | 155.0 | 165.0 |  | 169.5 |  | 0 |
| 39.8-40.25 | 60.9R | R | Raise | (4) Trench Fill Revt. | 155.0 | 165.0 |  | 169.5 |  | 0 |
| 42.7-43.05 | 62.93R | R | Raise | (1) Dike | 160.0 | 165.0 |  | 171.0 |  | 0 |
| 43.1-43.4 | 62.97-63.62R | D | Raise | (1) Dike | 160.0 | 165.0 |  | 171.5 |  | 0 |
| 43.65 | 63.50R | D | Extend | (1) Dike | 160.0 | 160.0 |  | 172.0 |  | 250 |
| 43.8 | 63.62R | D | Extend | (1) Dike | 160.0 | 160.0 |  | 172.0 |  | 300 |
| 44 | NEW-R | D | NEW | (3) New Dike Const. | 160.0 | N/A |  | 165.0 |  | 460 |
| 44.2 | NEW-R | D | NEW | (3) New Dike Const. | 160.0 | N/A |  | 165.0 |  | 460 |
| 44.4 | NEW-R | D | NEW | (3) New Dike Const. | 160.0 | N/A |  | 165.0 |  | 520 |
| 44.6 | NEW-R | D | NEW | (3) New Dike Const. | 160.0 | N/A |  | 165.0 |  | 600 |
| 46.25 | 68.8R | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.0 |  | 225 |
| 46.35 | 69.8R | R | Extend | (1) Dike | 150.0 | 150.0 |  | 173.0 |  | 365 |
| 46.6 | 68.6L | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.0 |  | 275 |
| 46.9 | 68.8L | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.0 |  | 300 |
| 48.2-48.8 | 72.0L | R | Extend | (4) Trench Fill Revt. | 158.0 | 172.0 |  | 173.5 |  | 300 |
| 48.7 | 70.78R | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.5 |  | 200 |
| 48.8 | 70.91R | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.5 |  | 200 |
| 48.9 | 71.03R | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.5 |  | 200 |
| 49 | 71.17R | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.5 |  | 200 |
| 49.15 | 71.37R | D | Extend | (1) Dike | 155.0 | 155.0 |  | 173.5 |  | 50 |
| POOL 3 – REACH 1 |  |  |  |  |  |  |  |  |  |  |
| 61.18 | 83.06L | D | Raise | (1) Dike | 170.0 | 180.0 | 185.4 |  |  | 0 |

*1 D-Dike; R-Revetment; D/P-Pile Dike; LH-L Head; BW-Bendway Weir.

*2 See Plate 1 “Typical Sections of River Training Structures” for details (Type #).

Table E-5 (Continued)

### Structures to be Raised, Extended, or Added in Arkansas

To Support a 12-Foot Navigation Channel

| Navigation Mile | Structure Number | Existing Type (*1) | Construction Activity | Typical Section (*2) (Type #) Description | Existing Ground Elevation FT-NGVD | Existing Top Elevation FT-NGVD | Proposed Top Elevation FT-NGVD |  | Delta Length FT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POOL 4 – REACH 2 |  |  |  |  |  |  |  |  |  |  |
| 79.3 | NEW-L | D | NEW | (3) New Dike Const. | 180.0 | N/A | 200.0 |  | 250 |  |
| 79.49 | NEW-L | D | NEW | (3) New Dike Const. | 180.0 | N/A |  | 200.0 |  | 250 |
| 79.68 | NEW-L | D | NEW | (3) New Dike Const. | 180.0 | N/A |  | 200.0 |  | 250 |
| 79.87 | NEW-L | D | NEW | (3) New Dike Const. | 180.0 | N/A |  | 200.0 |  | 250 |
| POOL 5 – REACH 2 |  |  |  |  |  |  |  |  |  |  |
| 96.2 - 98.4 | NEW-R | R | NEW | (5) Dumped Stone | 205.0 | N/A |  | 219.0 |  | 11900 |
| 96 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.1 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.2 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.3 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.4 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.5 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.6 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.7 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.8 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 96.9 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.1 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.2 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.3 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.4 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.5 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.6 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.7 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.8 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 97.9 | NEW-R | D | NEW | (3) New Dike Const. | 205.0 | N/A |  | 215.5 |  | 350 |
| 100.76 | NEW-R | BW | NEW | (6) Bendway Weir | 185.0 | N/A |  | 195.0 |  | 300 |
| 100.85 | NEW-R | BW | NEW | (6) Bendway Weir | 185.0 | N/A |  | 195.0 |  | 300 |
| 100.95 | NEW-R | BW | NEW | (6) Bendway Weir | 185.0 | N/A |  | 195.0 |  | 300 |
| 101.4 | NEW-R | BW | NEW | (6) Bendway Weir | 188.0 | N/A |  | 195.0 |  | 300 |
| 101.14 | NEW-R | BW | NEW | (6) Bendway Weir | 190.0 | N/A |  | 195.0 |  | 300 |
| 101.23 | NEW-R | BW | NEW | (6) Bendway Weir | 190.0 | N/A |  | 195.0 |  | 200 |
| 101.7 | 148.1L | LH | Raise | (1) Dike | 193.0 | 212.0 |  | 217.8 |  | 0 |
| 101.9 | 148.3L | LH | Raise | (1) Dike | 194.0 | 208.5 |  | 217.8 |  | 0 |
| 102.1 | 148.4L | LH | Raise | (1) Dike | 190.0 | 204.5 |  | 217.8 |  | 0 |
| 102.4 | 148.7L | LH | Raise | (1) Dike | 191.6 | 209.3 |  | 217.9 |  | 0 |

*1 D-Dike; R-Revetment; D/P-Pile Dike; LH-L Head; BW-Bendway Weir.

*2 See Plate 1 “Typical Sections of River Training Structures” for details (Type #).

Table E-5 (Continued)

### Structures to be Raised, Extended, or Added in Arkansas

To Support a 12-Foot Navigation Channel

| Navigation Mile | Structure Number |  | Existing Type (*1) |  | Construction Activity |  | Typical Section (*2) (Type #) Description | Existing Ground Elevation FT-NGVD | Existing Top Elevation FT-NGVD | Proposed Top Elevation FT-NGVD | Delta Length FT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### POOL 7 – REACH 3 |  |  |  |  |  |  |  |  |  |  |  |
| 126.9 | 173.2L |  |  | D | Extend | (1) Dike |  | 235.2 | 242.9 | 251.0 | 150 |
| 127.1 | 173.4L |  |  | LH | Raise | (1) Dike |  | 214.8 | 239.3 | 251.0 | 0 |
| 127.3 | 173.6L |  |  | LH | Raise | (1) Dike |  | 206.3 | 236.3 | 251.0 | 0 |
| 127.45 | 173.7L |  |  | LH | Raise | (1) Dike |  | 220.6 | 236.3 | 251.0 | 0 |
| 142.51 | 188.0R |  |  | LH | Raise | (1) Dike |  | 239.3 | 251.4 | 254.3 | 0 |
| 142.51 | 188.0R |  |  | D | Extend | (1) Dike |  | 239.3 | 245.4 | 254.3 | 50 |
| 142.69 | NEW-R |  |  | D | NEW | (3) New Dike Const. |  | 238.0 | N/A | 256.2 | 500 |
| 142.9 | 188.4R |  |  | D | Extend | (1) Dike |  | 229.0 | 252.9 | 254.6 | 400 |
| 143.05 | 188.6R |  |  | D | Extend | (1) Dike |  | 229.3 | 252.6 | 254.6 | 300 |
| 143.2 | 188.7R |  |  | D | Extend | (1) Dike |  | 241.6 | 250.0 | 254.6 | 200 |
| 145.09 | NEW-L |  |  | D | NEW | (3) New Dike Const. |  | 240.0 | N/A | 255.3 | 375 |
| 145.25 | NEW-L |  |  | D | NEW | (3) New Dike Const. |  | 240.0 | N/A | 255.3 | 375 |
| 145.4 | NEW-L |  |  | D | NEW | (3) New Dike Const. |  | 240.0 | N/A | 255.3 | 400 |
| 145.55 | NEW-L |  |  | D | NEW | (3) New Dike Const. |  | 240.0 | N/A | 255.3 | 400 |
| 145.7 | 191.2L |  |  | D | Extend | (1) Dike |  | 240.5 | 252.7 | 255.5 | 200 |
| 145.9 | 191.4L |  |  | D | Extend | (1) Dike |  | 241.5 | 251.2 | 255.6 | 200 |
| 146.2 | 191.7L |  |  | D | Extend | (1) Dike |  | 239.6 | 251.7 | 256.0 | 300 |
| 146.52 | 191.9L |  |  | D | Extend | (1) Dike |  | 229.7 | 252.7 | 256.0 | 300 |
| 146.88 | 192.3L |  |  | D | Extend | (1) Dike |  | 239.8 | 245.7 | 256.0 | 300 |
| 147.1 | 192.6L |  |  | D | Extend | (1) Dike |  | 240.7 | 244.7 | 256.0 | 200 |
| 147.28 | 192.8L |  |  | D | Extend | (1) Dike |  | 239.6 | 244.7 | 256.1 | 200 |
| 147.52 | 193.0L |  |  | D | Extend | (1) Dike |  | 239.7 | 245.7 | 256.2 | 200 |
| ###### POOL 8 – REACH 3 |  |  |  |  |  |  |  |  |  |  |  |
| 164.6 | 209.4L | D |  |  | Raise | (1) Dike |  | 255.8 | 266.5 | 270.0 | 0 |
| 164.9 | 209.7L | LH |  |  | Raise | (1) Dike |  | 230.1 | 250.1 | 270.0 | 0 |
| 169.27 | 217.3L | D |  |  | Extend | (1) Dike |  | 253.8 | 260.8 | 273.0 | 100 |
| 169.4 | 217.4L | D |  |  | Extend | (1) Dike |  | 250.8 | 260.8 | 273.0 | 100 |
| 169.5 | 217.5L | D |  |  | Extend | (1) Dike |  | 257.9 | 260.9 | 273.0 | 50 |
| 169.6 | 217.6L | D |  |  | Extend | (1) Dike |  | 256.9 | 260.9 | 273.0 | 150 |
| 169.8 | 217.7L | D |  |  | Extend | (1) Dike |  | 260.0 | 271.9 | 273.0 | 250 |
| 170.1 | 218.1L | D/P |  |  | Extend | (2) Pile Dike |  | 260.4 | 268.4 | 273.0 | 250 |

*1 D-Dike; R-Revetment; D/P-Pile Dike; LH-L Head; BW-Bendway Weir.*2 See Plate 1 “Typical Sections of River Training Structures” for details (Type #).

*2 See Plate 1 “Typical Sections of River Training Structures” for details (Type #).

### Table A-10 (Continued)

### Structures to be Raised, Extended, or Added in Arkansas

| Navigation Mile | Structure Number |  | Existing Type (*1) | Construction Activity | Typical Section (*2) (Type #) Description | Existing Ground Elevation FT-NGVD |  | Existing Top Elevation FT-NGVD |  |  |  | Proposed Top Elevation FT-NGVD |  | Delta Length FT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POOL 9 – REACH 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 181.5 | 229.4R | D |  | Extend | (1) Dike | 266.7 |  |  | 284.4 |  |  |  | 288.0 |  | 150 |
| 181.8 | 229.7R | D |  | Extend | (1) Dike | 265.5 |  |  | 283.5 |  |  |  | 288.0 |  | 150 |
| 182.2 | 230.0R | D |  | Extend | (1) Dike | 273.0 |  |  | 282.0 |  |  |  | 288.0 |  | 150 |
| 184.6 | 232.1R | D |  | Extend | (1) Dike | 278.3 |  |  | 282.3 |  |  |  | 288.0 |  | 100 |
| 184.9 | 232.4R | D/P |  | Extend | (2) Pile Dike | 270.8 |  |  | 284.3 |  |  |  | 288.0 |  | 100 |
| 185.2 | 232.7R | D/P |  | Extend | (2) Pile Dike | 273.8 |  |  | 284.3 |  |  |  | 288.0 |  | 100 |
| 185.3 | 232.8R | D/P |  | Extend | (2) Pile Dike | 261.3 |  |  | 282.1 |  |  |  | 288.0 |  | 100 |
| 186.95 - 187.05 | 234.5R | VD |  | Raise | (1) Dike | 275.0 |  |  | 286.0 |  |  |  | 289.0 |  | 0 |
| 187.3 | 234.9R | D |  | Raise | (1) Dike | 272.6 |  |  | 286.0 |  |  |  | 289.0 |  | 0 |
| 189.8 - 190.5 | 237.9L | R |  | Raise | (1) Dike | 279.0 |  |  | 287.0 |  |  |  | 290.2 |  | 0 |
| DARDANELLE – REACH 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 236.3 | 288.4R | D |  | Extend | (1) Dike |  | 330.3 |  |  | 338.7 | 340.0 |  |  | 100 |  |
| 236.75 - 237.42 | 290.0R | R |  | Raise | (4) Trench Fill Revet. |  | 317.4 |  |  | 339.0 | 340.0 |  |  | 0 |  |
| 238.95 | 291.3L | D |  | Raise | (1) Dike |  | 326.0 |  |  | 336.3 | 340.0 |  |  | 0 |  |
| 239.28 | 291.6L | D |  | Extend | (1) Dike |  | 329.3 |  |  | 336.5 | 340.0 |  |  | 50 |  |
| 239.8 | 292.3L | D |  | Extend | (1) Dike |  | 329.6 |  |  | 338.3 | 340.0 |  |  | 50 |  |
| 240.15 | 292.5L | D |  | Extend | (1) Dike |  | 321.4 |  |  | 335.0 | 340.0 |  |  | 100 |  |
| 240.46 | 292.9L | D |  | Extend | (1) Dike |  | 333.0 |  |  | 339.0 | 340.0 |  |  | 100 |  |
| 240.78 | 293.1L | D |  | Extend | (1) Dike |  | 335.6 |  |  | 337.9 | 340.3 |  |  | 100 |  |
| 241.05 | 293.4L | D |  | Extend | (1) Dike |  | 327.6 |  |  | 337.3 | 340.5 |  |  | 100 |  |
| 241.05 - 241.5 | 293.7L | R |  | Extend | (4) Trench Fill Revet. |  | 325.5 |  |  | 335.5 | 340.5 |  |  | 100 |  |
| 241.5 | 293.7R | D |  | Extend | (1) Dike |  | 328.6 |  |  | 337.0 | 340.5 |  |  | 100 |  |
| 241.73 | 293.9R | D |  | Extend | (1) Dike |  | 327.5 |  |  | 340.8 | 342.0 |  |  | 200 |  |
| 242 | 294.1R | D |  | Extend | (1) Dike |  | 330.9 |  |  | 337.8 | 342.0 |  |  | 200 |  |
| 242.12 | 294.3R | D |  | Extend | (1) Dike |  | 330.0 |  |  | 339.8 | 342.0 |  |  | 200 |  |
| 242.28 | 294.4R | D |  | Extend | (1) Dike |  | 330.8 |  |  | 342.5 | 342.0 |  |  | 200 |  |

### To Support a 12-Foot Navigation Channel

Table E-5 (Continued)

### Structures to be Raised, Extended, or Added in Arkansas

To Support a 12-Foot Navigation Channel

| Navigation Mile | Structure Number | Existing Type (*1) | Construction Activity | Typical Section (*2) (Type #) Description | Existing Ground Elevation FT-NGVD | Existing Top Elevation FT-NGVD | Proposed Top Elevation FT-NGVD | Delta Length FT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### OZARK – REACH 4 |  |  |  |  |  |  |  |  |
| 275.15 | 327.6L | D | Raise | (1) Dike | 361.3 | 373.3 | 374.5 | 0 |
| 275.25 | 327.7R | D | Raise | (1) Dike | 354.8 | 364.2 | 374.5 | 0 |
| 275.3 | 327.9L | D | Raise | (1) Dike | 356.0 | 357.0 | 374.5 | 0 |
| 275.49 | 328.0L | D/R | Raise | (1) Dike | 355.8 | 364.8 | 374.5 | 0 |
| 275.55 | 328.0R | D | Raise | (1) Dike | 362.7 | 366.8 | 374.6 | 0 |
| 275.85 | 328.4R | D | Raise | (1) Dike | 356.8 | 357.0 | 374.8 | 0 |
| 276.2 | 328.7R | D | Raise | (1) Dike | 356.8 | 369.0 | 375.0 | 0 |
| 279.58 | 333.6R | D | Extend | (1) Dike | 352.8 | 373.3 | 376.0 | 200 |
| 279.8 | 333.8R | D | Extend | (1) Dike | 361.8 | 373.2 | 376.3 | 200 |
| 280.05 | 333.9R | D | Extend | (1) Dike | 357.8 | 373.8 | 376.8 | 200 |
| 278.9 - 280.25 | 334.0L | R | Raise | (1) Dike | 362.1 | 371.8 | 376.5 | 0 |
| 280.2 | 334.0R | D | Extend | (1) Dike | 358.8 | 373.6 | 376.6 | 100 |
| 280.3 - 280.6 | 334.3R | R | Extend | (4) Trench Fill Revet. | 361.3 | 374.0 | 376.7 | 400 |
| 280.48 | 334.3L | D | Extend | (1) Dike | 356.8 | 372.2 | 376.7 | 100 |
| 280.67 | NEW-L | D | NEW | (3) New Dike Const. | 365.0 | N/A | 376.9 | 700 |
| 280.86 | NEW-L | D | NEW | (3) New Dike Const. | 365.0 | N/A | 377.0 | 450 |
| 283.76 | NEW-R | D | NEW | (3) New Dike Const. | 365.0 | N/A | 378.3 | 100 |
| 283.35 - 283.8 | 337.5L | R | Raise | (1) Dike | 359.6 | 374.0 | 378.3 | 0 |
| 283.95 | NEW-R | D | NEW | (3) New Dike Const. | 365.0 | N/A | 378.5 | 100 |
| 284.1 | NEW-R | D | NEW | (3) New Dike Const. | 365.0 | N/A | 378.5 | 100 |
| 285.4 | 339.4R | LH | Raise | (1) Dike | 360.6 | 375.2 | 381.0 | 0 |
| 285.4 | NEW-L | D | NEW | (3) New Dike Const. | 365.0 | N/A | 379.0 | 400 |
| 285.65 | 339.5L | D | Raise | (1) Dike | 357.5 | 374.6 | 379.1 | 0 |
| 285.9 | 339.7L | D | Raise | (1) Dike | 359.5 | 373.0 | 379.2 | 0 |

*1 D-Dike; R-Revetment; D/P-Pile Dike; LH-L Head; BW-Bendway Weir.

*2 See Plate 1 “Typical Sections of River Training Structures” for details (Type #).

5.  Notched Dikes/Revetments

Table E-6 shows the location of the notched dikes and revetments along the MKARNS.

| Table E-6 Notched Dikes/Revetments |  |  |
| --- | --- | --- |
| Arkansas River Notching Projects Completed as of 2/22/02 |  |  |
| Project Type | Navigation Mile | River Pool |
| Dike Notch Revetment Notch* Revetment Notch Revetment Notch* Dike Notch Revetment Notch Revetment Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Revetment Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Dike Notch Revetment Notch* Revetment Notch* Revetment Notch* Dike Notch Revetment Notch* | 78.7 left bank 91.0 left bank 98.5 left bank 104.1 right bank 106.5 left bank 107 left bank 107.2 left bank 116 right bank 116.3 right bank 116.6 right bank 116.8 right bank 117.1 right bank 117.25 right bank 117.4 right bank 117.55 right bank 117.7 right bank 120.3 right bank 120.4 left bank 120.5 right bank 120.65 right bank 120.8 right bank 120.8 left bank 122.7 left bank 125.7 right bank 146.15 right bank 146.3 right bank 146.6 right bank 150.9 left bank 151.3 left bank 151.9 right bank 152.1 right bank 152.4 right bank 152.6 right bank 152.9 right bank 153.2 right bank 153.45 right bank 154 right bank 154.2 right bank 154.5 right bank 156.75 right bank 156.9 right bank 157 right bank 241.1 left bank 241.2 left bank | Pool 4 Pool 5 Pool 5 Pool 5 Pool 5 Pool 5 Pool 5 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 6 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 7 Pool 8 Pool 8 Pool 8 Pool 10 Pool 10 |
| * Indicates shallow, narrow notch for fish passage only, not boats |  |  |

6.  Maintenance Dredging History

Tables E-7 and E-8 show the historic maintenance dredging quantities on the Verdigris and Arkansas Rivers in Oklahoma.

| Table E-7 Maintenance Dredging History on the Verdigris River |  |  |  |
| --- | --- | --- | --- |
| Navigation Mile | Site No. | Quantities (cy) | Location |
| 444.6 – 445 | 18A | 957,050 | Bird Creek/Verdigris River & Port of Catoosa Area, NM 444 to 445 |
| 444 - 445 | 18B |  | Bird Creek/Verdigris River & Port of Catoosa Area, NM 444 to 445 |
| 421.6 - 422.2 | 18C | 61,570 | Above L&D 18, NM 421 to 422 |
| 420.8 -421.6 | 17A | 387,400 | Below L&D 18 Good Hope Area, NM 420 to 421 |
| 401.6 - 402.6 | 17B | 292,960 | Above L&D 17, NM 401 to 403 |
| 400.5 - 401 | 16A & A-1 | 384,720 | Below L&D 17 & Confluence of Spillway Channel, NM 400 to 401 |
| 395 - 395.5 | 16B & C | 621,950 | Three Forks, Verdigris River Area |

| Table E-8 Maintenance Dredging History on the Arkansas River |  |  |  |
| --- | --- | --- | --- |
| Navigation Mile | Site No. | Quantities (cy) | Location |
| 394 - 395 | 16B, C & 16D | 392,410 | Three Forks, Arkansas River Area |
| 393 - 395 | 16E & G | ------- | Three Forks, Arkansas River Area |
| 392.8 - 393.3 | 16F & G | 104,892 | Three Forks, Highway 62 Bridge Area |
| 353 - 356 | 15A & A-1 | 117,210 | Stoney Point & Confluence of Canadian River Area, NM 353 to 357 |
| 348 - 349.5 | 15B & B-1 | ------- | Sandtown Bottom, Tamaha Area, NM 348 to 350 |
| 6.5 - 7.5 SBC | 15C & C-1 | ------- | Sans Bois Creek |
| 8 - 11 SBC | 15D, E & 15F | ------- | Sans Bois Creek |
| 318.3 - 319.1 | 13A | 379,445 | Below L&D 14, NM 318 to 319 |
| 315 - 317.2 | 13B |  | Pool 13 Peno Point Area, NM 315 to 317 |
| 311.8 - 313.9 | 13C | 513,227 | Pool 13 Camp Creek Area, NM 312 to 314 |
| 308.8 - 311 | 13D | 288,400 | Right descending Ark River @ Poteau River |
| 0.0 – 2.0 PR | 13E | ------- | Adjacent to Poteau River Turning Basin |

7.  Future Anticipated Dredging Needs

Tables E-9 and E-10 show the future anticipated dredging needs on the Verdigris and Arkansas rivers in Oklahoma from 2003 to 2023.

| Table E-9 Future Anticipated Dredging on the Verdigris River |  |  |  |
| --- | --- | --- | --- |
| Navigation Mile | Site No. | Quantities (cy) | Location |
| 444.6 - 445 | 18A | 600,000 | Bird Creek/Verdigris River & Port of Catoosa Area |
| 444 - 445 | 18B |  | Bird Creek/Verdigris River & Port of Catoosa Area |
| 421.6 - 422.2 | 18C | 100,000 | Above L&D 18 |
| 420.8 -421.6 | 17A | 500,000 | Below L&D 18 Good Hope |
| 401.6 - 402.6 | 17B | 300,000 | Above L&D 17 |
| 400.5 - 401 | 16A & A-1 | 400,000 | Below L&D 17 & Confluence of Spillway Channel |
| 395 - 395.5 | 16B & C | 900,000 | Three Forks, Verdigris River Area |

| Table E-10 Future Anticipated Dredging on the Arkansas River |  |  |  |
| --- | --- | --- | --- |
| Navigation Mile | Site No. | Quantities (cy) | Location |
| 393 - 395 | 16D & E | 1,300,000 | Three Forks, Arkansas River Area |
| 393 - 395 | 16G |  | Three Forks, Arkansas River Area |
| 392.8 - 393.3 | 16F & G | 500,000 | Three Forks, Highway 62 Bridge Area |
| 353 - 356 | 15A & A-1 | 100,000 | Stoney Point & Confluence of Canadian River Area |
| 348 - 349.5 | 15B & B-1 | 100,000 | Sandtown Bottom, Tamaha Area |
| 6.9 - 7.4 SBC | 15C & C-1 | 200,000 | Sans Bois Creek |
| 8 - 11 SBC | 15D, E & 15F | 300,000 | Sans Bois Creek |
| 318.3 - 319.1 | 13A | 200,000 | Below L&D 14 |
| 315 - 317.2 | 13B | 500,000 | Pool 13 Peno Point Area |
| 311.8 - 313.9 | 13C | 500,000 | Pool 13 Camp Creek Area |
| 308.6 - 312 | 13D | 300,000 | Right descending Ark River @ Poteau River |
| 0.0 - 2.0 Poteau River | 13E | 200,000 | Adjacent to Poteau River Turning Basin |

Beginning dredging in or around any areas described in this document depends on actual conditions.  Unexpected high flows may dictate when actual dredging is required for any given site.  Sediment load varies and bed load displacement may require dredging sooner than anticipated.

8.  Existing Levees

Table E-11 summarizes the levees located within the study area.

| Table E-11. Levees within the Arkansas River Navigation Study Area |  |
| --- | --- |
| MKARNS POOL | LEVEE |
| White River | Levee Mile 7, 8.5, and 9.2 |
| Pool 1 | Levee Mile 11 |
| Pool 2 | Pendleton Levee |
| Pool 2 | South Bend Levee |
| Pool 2 | Farelly Lake Levee District |
| Pool 2 | North Bank Levee Below Plum Bayou |
| Pool 2 | Jefferson County Levee District No. 3 |
| Pool 2 | Southeast Arkansas Levee District |
| Pool 3 | Jefferson County Levee District No. 3 |
| Pool 3 | Southeast Arkansas Levee District |
| Pool 3 | North Bank Levee Below Plum Bayou |
| Pool 3 | New Gascony Levee District |
| Pool 3 | Linwood - Auburn Levee District |
| Pool 4 | Linwood - Auburn Levee District |
| Pool 4 | Non-Overflow Structure (USACE) |
| Pool 4 | Tucker Lake Levee & Drainage District |
| Pool 4 | Plum Bayou Levee District |
| Pool 5 | T. A. Gibson Private Levee |
| Pool 5 | Plum Bayou Levee District |
| Pool 5 | Old River Drainage District |
| Pool 5 | Woodson Levee District |
| Pool 5 | Fourche Island Drainage District No. 2 |
| D. D. Terry Lake | Fourche Island Drainage District No. 2 |
| D. D. Terry Lake | Little Rock – Pulaski Drainage District No. 2 |
| D. D. Terry Lake | North Little Rock Levee & Floodwall |
| D. D. Terry Lake | W. D. Cammack Private Levee |
| Pool 7 | Roland Drainage District Levee |
| Pool 7 | Faulkner County Levee District No. 2 |
| Pool 7 | Perry County Levee No. 1 |
| Pool 7 | Faulkner County Levee District No. 1 |
| Pool 8 | Conway County Levee District Nos. 1, 6, 8 and 10 |
| Winthrop Rockefeller Lake | Conway County Levee District No. 1, 3 and 7 |
| Winthrop Rockefeller Lake | Pope County Levee & Drainage District No. 2 |
| Winthrop Rockefeller Lake | Galla Creek Levee |
| Winthrop Rockefeller Lake | Carden Bottoms Drainage District No. 2 |
| Winthrop Rockefeller Lake | Holla Bend Levee District No. 1 |
| Winthrop Rockefeller Lake | Point Bar Levee |
| Winthrop Rockefeller Lake | Flagg Lake Levee |
| Winthrop Rockefeller Lake | Dardanelle Drainage District Levee |
| Lake Dardanelle | Lower Hartman Bottom Levee |
| Lake Dardanelle | McLean Bottom Levee District No. 3 |
| Ozark Lake | Crawford County Levee District |
| Pool 13 | Crawford County Levee District |
| Pool 13 | Van Buren Levee District No. 1 |
| Pool 13 | Southern Enterprises Private Levee |
| Pool 13 | Fort Smith Levee Improvement District No. 1 |
| Chouteau Lake | Highway 51 |
| Chouteau Lake | Oxbow Island Park |
| Newt Graham | Rogers Point Park |
| Source: MKARNS Navigation Charts, 1997 |  |